1. Add AL, BL , where AL contains 80h, BL contains 60h,  how this will affect the flag register ?

Ans:  Effect on the Flag Register for ADD AL, BL

Initial Values:

AL = 80h

BL = 60h

Addition Operation:

AL + BL = 80h + 60h = E0h

Effect on Flags:

Carry Flag (CF): Not set (0), as the addition does not result in a carry beyond the most significant bit.

Zero Flag (ZF): Not set (0), as the result (E0h) is not zero.

Sign Flag (SF): Set (1), as the most significant bit of the result (E0h) is 1, indicating a negative number in signed arithmetic.

Overflow Flag (OF): Not set (0), as there is no signed overflow. Adding two positive numbers (interpreted in signed arithmetic) does not result in a negative number.

Auxiliary Carry Flag (AF): May or may not be set based on the 4-bit lower nibble addition; however, it is not commonly checked in such cases.

Parity Flag (PF): Set (1), as the number of 1s in the binary representation of E0h is even (11100000 has 3 zeros and 5 ones).

2.suppose AL and BL contains extended ASCII charecter to display the one that comes first which jump instruction should be used and why?

Ans:   2. Displaying the First Character in Extended ASCII

Initial Values:

AL and BL contain extended ASCII characters.

Task: To display the character that comes first in order (lower ASCII value).

Jump Instruction:

Use JC (Jump if Carry) or JL (Jump if Less).

Explanation:

Unsigned Comparison:

If the comparison is based on unsigned values, use the JC instruction:

Perform CMP AL, BL.

If AL < BL (unsigned), the Carry Flag (CF) will be set, and JC will jump to the appropriate code block.

Signed Comparison:

If the comparison is based on signed values, use the JL instruction:

Perform CMP AL, BL.

The JL instruction checks the Sign Flag (SF) and Overflow Flag (OF) to determine if AL < BL in signed arithmetic.

Why?

Jump instructions like JC and JL allow branching based on the result of the comparison, directing the program to handle the appropriate ASCII character based on the order.

Note: If the context specifies extended ASCII without signed interpretation, prefer JC for comparison.

CT-1 question

1. What is instruction prefetch in the 8086 microprocessors, and how does it work? 3

2. What are the advantages of assembly language over high-level languages? 3

3. What is the physical address for the memory location with a segment address of C0DEh

and an offset address of BEEFh?

1. Instruction Prefetch in the 8086 Microprocessors

Instruction prefetch is a mechanism used in the 8086 microprocessor to improve execution speed by fetching the next instruction(s) from memory before the current instruction finishes executing.

How it works:

The 8086 has a 6-byte instruction queue that prefetches instructions from memory.

While the EU (Execution Unit) is decoding or executing an instruction, the BIU (Bus Interface Unit) fetches the next instructions from memory and places them into the queue.

This overlap of instruction fetch and execution reduces wait time and improves overall processing efficiency.

If a jump or branch instruction occurs, the queue is flushed, and new instructions are fetched based on the new address.

2. Advantages of Assembly Language Over High-Level Languages

Efficiency:

Programs written in assembly language are highly efficient and provide direct control over hardware resources.

Precision:

It allows the programmer to use specific processor features and instructions, leading to precise and optimized code for critical tasks.

Hardware Interaction:

Assembly language is ideal for tasks requiring close interaction with hardware, such as device drivers or embedded systems, as it allows direct manipulation of memory and registers.

3. Physical Address Calculation

The physical address in the 8086 microprocessor is calculated using the formula:

Physical Address = (Segment Address × 16) + Offset Address

Physical Address = (Segment Address×16) + Offset Address

Given:

Segment Address = C0DEh

Offset Address = BEEFh

Steps:

Convert the segment address to its physical base address:

Base Address

=𝐶0𝐷𝐸ℎ×16

=𝐶0𝐷𝐸ℎ×10ℎ

=𝐶0𝐷𝐸0ℎ

Add the offset address:

Physical Address = 𝐶0𝐷𝐸0ℎ+𝐵𝐸𝐸𝐹ℎ=𝐶𝐶0𝐶𝐹ℎ

Physical Address=C0DE0h+BEEFh=CC0CFh

Final Physical Address: CC0CFh